Port knocking

/ip/firewall/filter add action=add-src-to-address-list address-list=888 address-list-timeout=30s chain=input dst-port=888 in-interface-list=WAN protocol=tcp

/ip/firewall/filter add action=add-src-to-address-list address-list=555 address-list-timeout=30s chain=input dst-port=555 in-interface-list=WAN protocol=tcp src-address-list=888

/ip/firewall/filter add action=add-src-to-address-list address-list=secured address-list-timeout=30m chain=input dst-port=222 in-interface-list=WAN protocol=tcp src-address-list=555

/ip/firewall/filter add action=accept chain=input in-interface-list=WAN src-address-list=secured

Blacklist

/ip/firewall/filter add action=drop chain=input disabled=yes in-interface-list=WAN src-address-list=blacklist

/ip/firewall/filter add action=add-src-to-address-list address-list=blacklist address-list-timeout=1000m chain=input disabled=yes dst-port=666 in-interface-list=WAN protocol=tcp

/ip/firewall/filter add action=add-src-to-address-list address-list=blacklist address-list-timeout=1m chain=input disabled=yes dst-port=21,22,23,8291,10000-60000 in-interface-list=WAN protocol=tcp src-address-list=!secured